DIGITAL SYSTEM DESIGN APPLICATION – EHB 436E

Experiment IV

Yiğit Bektaş GÜRSOY

040180063

Class Lecturer: Sıddıka Berna Örs Yalçın

Class Assistant:
Serdar Duran
Yasin Fırat Kula
Mehmet Onur Demirtürk

- HALF ADDER

- Verilog Code

module HA(

input X,

input Y,

output COUT,

output S

);

assign COUT = X & Y; //CARRY

assign S = X ^ Y ; //SUM

endmodule

- Testbench Code

//HALF ADDER TEST BENCH

module HA_tb();

reg X;

reg Y;

wire COUT;

wire SUM;

HA DUT(.X(X),

.Y(Y),

.COUT(COUT),

.S(SUM)

);

initial

begin

X = 1'b0 ; Y = 1'b0;

#10 X = 1'b0 ; Y = 1'b1;

#10 X = 1'b1 ; Y = 1'b0;

#10 X = 1'b1 ; Y = 1'b1;

#10 $finish;

end

endmodule

- RTL Schematic

- Technology Schematic

- There are 2 LUT2 in technology schematic.

- Combinational Delay

- The maximum delay in my circuit is 6.968ns.

- FULL ADDER

- Verilog Code

module FA(

input X,

input Y,

input CIN,

output COUT,

output S

);

wire signal_1;

wire signal_2;

wire signal_3;

HA halfadder1 ( .X(X),

.Y(Y),

.COUT(signal_1),

.S(signal_2)

);

HA halfadder2 ( .X(signal_2),

.Y(CIN),

.COUT(signal_3),

.S(S)

);

assign COUT = signal_3 | signal_1;

endmodule

- Testbench Code

//FULL ADDER TEST BENCH

module FA_tb();

reg X;

reg Y;

reg CIN;

wire COUT;

wire SUM;

FA DUT(.X(X),

.Y(Y),

.CIN(CIN),

.COUT(COUT),

.S(SUM)

);

initial

begin

X = 1'b0 ; Y = 1'b0; CIN = 1'b0;

#10 X = 1'b0 ; Y = 1'b0; CIN = 1'b1;

#10 X = 1'b0 ; Y = 1'b1; CIN = 1'b0;

#10 X = 1'b0 ; Y = 1'b1; CIN = 1'b1;

#10 X = 1'b1 ; Y = 1'b0; CIN = 1'b0;

#10 X = 1'b1 ; Y = 1'b0; CIN = 1'b1;

#10 X = 1'b1 ; Y = 1'b1; CIN = 1'b0;

#10 X = 1'b1 ; Y = 1'b1; CIN = 1'b1;

#10 $finish;

end

endmodule

- Full-adder Circuit

- RTL Schematic

- Technology Schematic

- There are 2 LUT3 in technology schematic.

- Combinational Delay

- The maximum delay in my circuit is 6.9ns.

- RIPPLE CARRY ADDER

- Verilog Code

module RCA(

input [3:0]X,

input [3:0]Y,

input CIN,

output COUT,

output [3:0]S

);

wire c1,c2,c3,c4;

FA fulladder1(

.X(X[0]),

.Y(Y[0]),

.CIN(CIN),

.COUT(c1),

.S(S[0])

);

FA fulladder2(

.X(X[1]),

.Y(Y[1]),

.CIN(c1),

.COUT(c2),

.S(S[1])

);

FA fulladder3(

.X(X[2]),

.Y(Y[2]),

.CIN(c2),

.COUT(c3),

.S(S[2])

);

FA fulladder4(

.X(X[3]),

.Y(Y[3]),

.CIN(c3),

.COUT(COUT),

.S(S[3])

);

endmodule

- Testbench Code

//RIPPLE CARRY ADDER TEST BENCH

module RCA_tb();

reg [3:0]X;

reg [3:0]Y;

reg CIN;

wire COUT;

wire [3:0]SUM;

RCA DUT(.X(X),

.Y(Y),

.CIN(CIN),

.COUT(COUT),

.S(SUM)

);

initial

begin

X = 4'b0000 ; Y = 4'b0000; CIN = 1'b0;

#10 X = 4'b1110 ; Y = 4'b0011; CIN = 1'b1;

#10 X = 4'b0110 ; Y = 4'b1100; CIN = 1'b0;

#10 X = 4'b0011 ; Y = 4'b1010; CIN = 1'b1;

#10 X = 4'b1000 ; Y = 4'b1111; CIN = 1'b0;

#10 X = 4'b1111 ; Y = 4'b0101; CIN = 1'b1;

#10 X = 4'b1001 ; Y = 4'b1100; CIN = 1'b0;

#10 X = 4'b1101 ; Y = 4'b1111; CIN = 1'b1;

#10 $finish;

end

endmodule

- RTL Schematic

- Technology Schematic

- There are 4 LUT5 and 2 LUT3 in technology schematic.

- Combinational Delay

- The maximum delay in my circuit is 7.819ns.

- RIPPLE CARRY ADDER WITH GENERATE FOR

- Verilog Code

module parametric_RCA #( parameter SIZE=4 )

(

input [SIZE-1:0]X,

input [SIZE-1:0]Y,

input CIN,

output COUT,

output [SIZE-1:0]S

);

wire [SIZE:0]signal;

assign signal[0] = CIN;

genvar i;

generate

for(i = 0; i < SIZE; i = i + 1)

begin: generated_FA

FA fulladder(

X[i],

Y[i],

signal[i],

signal[i+1],

S[i]

);

end

endgenerate

assign COUT = signal[SIZE];

endmodule

- 4-bit wide Testbench Code

module parametric_RCA_tb();

parameter SIZE = 4 ;

reg [SIZE-1:0]X;

reg [SIZE-1:0]Y;

reg CIN;

wire COUT;

wire [SIZE-1:0]S;

parametric_RCA #(

.SIZE(SIZE)

)

RCA1(.X(X),

.Y(Y),

.CIN(CIN),

.COUT(COUT),

.S(S));

initial

begin

X = 4'b0011 ; Y = 4'b0011; CIN = 1'b1;

#10 X = 4'b0110 ; Y = 4'b1000; CIN = 1'b0;

#10 X = 4'b0011 ; Y = 4'b1000; CIN = 1'b1;

#10 X = 4'b0011 ; Y = 4'b1100; CIN = 1'b0;

#10 X = 4'b0001 ; Y = 4'b0111; CIN = 1'b1;

#10 $finish;

end

endmodule

- 4-bit wide simulation result

- 8-bit wide testbench code

module parametric_RCA_tb();

parameter SIZE = 4 ;

reg [SIZE-1:0]X;

reg [SIZE-1:0]Y;

reg CIN;

wire COUT;

wire [SIZE-1:0]S;

parametric_RCA #(

.SIZE(SIZE)

)

RCA1(.X(X),

.Y(Y),

.CIN(CIN),

.COUT(COUT),

.S(S));

initial

begin

X = 8'b10001110 ; Y = 8'b10100011; CIN = 1'b1;

#10 X = 8'b01010110 ; Y = 8'b00111100; CIN = 1'b0;

#10 X = 8'b01100011 ; Y = 8'b10001010; CIN = 1'b1;

#10 X = 8'b00001000 ; Y = 8'b00101111; CIN = 1'b0;

#10 X = 8'b01011111 ; Y = 8'b10000101; CIN = 1'b1;

#10 $finish;

end

endmodule

- 8-bit wide simulation result

- RTL Schematic

- 4-bit wide RCA with generate for

- 8-bit wide RCA with generate for

- Technology Schematic

- 4-bit wide RCA with generate for

4-bit wide RCA with generate for includes 4 LUT5s and 2 LUT3.

- 8-bit wide RCA with generate for

8-bit wide RCA with generate for includes 8 LUT5s and 3 LUT3.

Our 4-bit RCA circuits are very similar to each other. In our 8-bit RCA circuit, the use of LUT5 and LUT3 is more.

- CARRY LOOKAHEAD ADDER

module CGA ( //carry , generate , propagate

input [3:0]P,

input [3:0]G,

input CIN,

output [3:0]COUT

);

assign COUT[0] = G[0] | (P[0] & CIN);

assign COUT[1] = G[1] | (G[0] & P[1]) | (P[1] & P[0] & CIN);

assign COUT[2] = G[2] | (G[1] & P[2]) | (G[0] & P[2] & P[1]) | ( P[2] & P[1] & P[0] & CIN);

assign COUT[3] = G[3] | (G[2] & P[3]) | (G[1] & P[3] & P[2]) | (G[0] & P[3] & P[2] & P[1]) | (P[3] & P[2] & P[1] & P[0] & CIN);

endmodule

module CLA (

input [3:0]X,

input [3:0]Y,

input CIN,

output COUT,

output [3:0]S

);

wire [3:0]P;

wire [3:0]G;

wire [3:0]C;

assign P = X ^ Y;

assign G = X & Y;

CGA CGA(

.P(P),

.G(G),

.CIN(CIN),

.COUT(C)

);

assign COUT = C[3];

assign S[0] = P[0] ^ CIN;

assign S[1] = P[1] ^ C[0];

assign S[2] = P[2] ^ C[1];

assign S[3] = P[3] ^ C[2];

endmodule

Verilog Code

- Testbench Code

//CLA TEST BENCH

module CLA_tb();

reg [3:0]X;

reg [3:0]Y;

reg CIN;

wire COUT;

wire [3:0]S;

CLA DUT(.X(X),

.Y(Y),

.CIN(CIN),

.COUT(COUT),

.S(S)

);

initial

begin

X = 4'b0000 ; Y = 4'b0000; CIN = 1'b0;

#10 X = 4'b1110 ; Y = 4'b0011; CIN = 1'b1;

#10 X = 4'b0110 ; Y = 4'b1100; CIN = 1'b0;

#10 X = 4'b0011 ; Y = 4'b1010; CIN = 1'b1;

#10 X = 4'b1000 ; Y = 4'b1111; CIN = 1'b0;

#10 X = 4'b1111 ; Y = 4'b0101; CIN = 1'b1;

#10 X = 4'b1001 ; Y = 4'b1100; CIN = 1'b0;

#10 X = 4'b1101 ; Y = 4'b1111; CIN = 1'b1;

#10 $finish;

end

endmodule

- RTL Schematic

- Technology Schematic

- There are 2 LUT3 and 4 LUT5 in technology schematic.

- Combinational Delay

- The maximum delay in my circuit is 7.819ns. We can say that most of the combinational delays are either close to 7ns or higher than 7ns.

- ADDER-SUBSTRACTOR CIRCUIT WITH OVERFLOW DETECTION

- Verilog Code

module ADD_SUB

(

input [3:0]A,

input [3:0]B,

input CIN,

output [3:0]SUM,

output COUT,

output V

);

wire fa1_in, fa2_in, fa3_in, fa4_in;

wire fa1_cout, fa2_cout, fa3_cout;

assign fa1_in = CIN ^ B[0];

assign fa2_in = CIN ^ B[1];

assign fa3_in = CIN ^ B[2];

assign fa4_in = CIN ^ B[3];

FA FA1

(

.X(A[0]),

.Y(fa1_in),

.CIN(CIN),

.COUT(fa1_cout),

.S(SUM[0])

);

FA FA2

(

.X(A[1]),

.Y(fa2_in),

.CIN(fa1_cout),

.COUT(fa2_cout),

.S(SUM[1])

);

FA FA3

(

.X(A[2]),

.Y(fa3_in),

.CIN(fa2_cout),

.COUT(fa3_cout),

.S(SUM[2])

);

FA FA4

(

.X(A[3]),

.Y(fa4_in),

.CIN(fa3_cout),

.COUT(COUT),

.S(SUM[3])

);

assign V = fa3_cout ^ COUT;

endmodule

- Testbench Code

module ADD_SUB_tb();

reg [3:0]A;

reg [3:0]B;

reg CIN;

wire COUT;

wire V;

wire [3:0] SUM;

ADD_SUB AS

(

.A(A),

.B(B),

.CIN(CIN),

.COUT(COUT),

.V(V),

.SUM(SUM)

);

initial

begin

A = 4'b0000; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0001; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0010; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0011; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0100; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0101; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0110; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0111; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1000; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1001; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1010; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1011; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1100; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1101; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1110; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b1111; B = 4'b0111; CIN = 1'b0;

#10 A = 4'b0000; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0001; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0010; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0011; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0100; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0101; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0110; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b0111; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1000; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1001; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1010; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1011; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1100; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1101; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1110; B = 4'b0111; CIN = 1'b1;

#10 A = 4'b1111; B = 4'b0111; CIN = 1'b1;

$finish;

end

endmodule

- Behavioral Simulation

- The values representing the yields at the outputs of the 3rd and 4th Full adders are fa3_cout and COUT, respectively.

- RTL Schematic

- Technology Schematic

- There are 1 LUT2, 1 LUT4, 2 LUT5 and 3 LUT6 in technology schematic.

- Combinational Delay

- The maximum delay in my circuit is 8.026ns

- In the photo above, the delays are arranged according to the output COUT. Delays are usually between 7ns and 8ns.

- RCA AND CLA COMPARISON WITH DONT TOUCH STRUCTURE ( OPTIONAL)

- Our results with our max delays were supposed to have less delay in terms of coding and structure in CLA, but since our program did its own optimization, the delays were equal in both cases. If we give the structure (*dont _touch = "true"*) to the wire parts, we can find the differences between them.

- Verilog Code (RCA and CLA)

module CGA ( //carry , generate , propagate

input [3:0]P,

input [3:0]G,

input CIN,

output [3:0]COUT

);

assign COUT[0] = G[0] | (P[0] & CIN);

assign COUT[1] = G[1] | (G[0] & P[1]) | (P[1] & P[0] & CIN);

assign COUT[2] = G[2] | (G[1] & P[2]) | (G[0] & P[2] & P[1]) | ( P[2] & P[1] & P[0] & CIN);

assign COUT[3] = G[3] | (G[2] & P[3]) | (G[1] & P[3] & P[2]) | (G[0] & P[3] & P[2] & P[1]) | (P[3] & P[2] & P[1] & P[0] & CIN);

endmodule

module CLA (

input [3:0]X,

input [3:0]Y,

input CIN,

output COUT,

output [3:0]S

);

(* dont_touch = "true" *) wire [3:0]P;

(* dont_touch = "true" *) wire [3:0]G;

(* dont_touch = "true" *) wire [3:0]C;

assign P = X ^ Y;

assign G = X & Y;

CGA CGA(

.P(P),

.G(G),

.CIN(CIN),

.COUT(C)

);

assign COUT = C[3];

assign S[0] = P[0] ^ CIN;

assign S[1] = P[1] ^ C[0];

assign S[2] = P[2] ^ C[1];

assign S[3] = P[3] ^ C[2];

endmodule

module RCA(

input [3:0]X,

input [3:0]Y,

input CIN,

output COUT,

output [3:0]S

);

(* dont_touch = "true" *)wire c1,c2,c3,c4;

FA fulladder1(

.X(X[0]),

.Y(Y[0]),

.CIN(CIN),

.COUT(c1),

.S(S[0])

);

FA fulladder2(

.X(X[1]),

.Y(Y[1]),

.CIN(c1),

.COUT(c2),

.S(S[1])

);

FA fulladder3(

.X(X[2]),

.Y(Y[2]),

.CIN(c2),

.COUT(c3),

.S(S[2])

);

FA fulladder4(

.X(X[3]),

.Y(Y[3]),

.CIN(c3),

.COUT(COUT),

.S(S[3])

);

endmodule

- Combinational Delay

RCA DELAY WITH DONT_TOUCH

CLA DELAY WITH DONT_TOUCH

- As you can see, the max delay in the RCA module that we made with the dont_touch structure was 12,441 ns, and the max delay in the CLA module was 10.636ns. There is a difference of about 2ns between them. Although both are summation circuits, the CLA module works faster than the RCA module. This is due to its design.